# Chuyên đề FCC: Cải cách Cấp phép Chòm sao NGSO và Cơ chế Xử lý Can nhiễu Suy giảm Băng thông

Tác giả / Cơ quan: Phòng Nghiên cứu Chính sách Phổ tần Quốc tế
Phân loại: Chiến lược & Chính sách
Tóm tắt: Phân tích các lệnh cải cách của FCC (FCC 17-122, FCC 24-117) chuyển từ giới hạn EPFD sang cơ chế bảo vệ suy giảm thông lượng 3% (throughput degradation) và quy định ký quỹ chống xí phần 5 triệu USD.

============================================================

# Note: FCC NGSO FSS Report and Order 2017

URL: https://docs.fcc.gov/public/attachments/DOC-346893A1.pdf (FCC news release, read 2026-10-19, index)

Document: Report and Order and FNPRM, FCC 17-122, IB Docket No. 16-408, adopted Sept 26, 2017.

Amended US Table of Allocations to accommodate NGSO/GSO in Ka-band (20/30 GHz).

Streamlined NGSO milestone rules: 50% deployment within 6 years, 100% within 9 years of license grant (replacing prior 100%-in-6-years rule); failure -> authorization reduced to number of satellites actually deployed.

Eliminated international geographic coverage requirements.

Adopted new default spectrum-sharing threshold mechanism absent coordination agreement between NGSO operators.

# Note: FCC NGSO sharing orders 2021-2026

URLs (index, read 2026-10-19):

https://www.telecomtv.com/content/access-evolution/fcc-authorizes-kuiper-satellite-constellation-full-fcc-document-39345/ (6% delta-T/T coordination trigger adopted in 2017 NGSO FSS Order for NGSO-NGSO; good-faith coordination required first; default = equal spectrum split during congestion; seniority by launch)

https://connectivitybusiness.com/news/government-legal/fcc-initiates-review-of-ngso-spectrum-sharing-regulations/ (Dec 2023 NPRM on NGSO sharing, SpaceX petition; 6% threshold borrowed from ITU in 2003 NPRM)

https://docs.fcc.gov/public/attachments/DOC-407457A1.pdf (FCC news release Nov 15, 2024: Second R&O; 3% time-weighted avg throughput degradation long-term; 0.4% absolute increase in link unavailability short-term)

https://www.lexology.com/library/detail.aspx?g=6554abb9-e969-42a7-8c47-56e62777c1ad (Second R&O: IB Docket 21-456, FCC 24-117, 39 FCC Rcd 12656, adopted Nov 5, 2024; declined aggregate limit; denied OneWeb recon on 10-year sunset)

https://www.tlp.law/2025/05/02/fcc-releases-adopted-nprm-proposing-to-update-spectrum-sharing-rules-for-satellite-broadband/ (NPRM adopted Apr 29, 2025: propose replacing EPFD with degraded-throughput in 10.7-12.7, 17.3-18.6, 19.7-20.2 GHz)

https://www.jdsupra.com/legalnews/fcc-modernizes-satellite-spectrum-5634424/ and https://www.lexology.com/library/detail.aspx?g=4476b010-c92c-4f71-a1e4-b8295933508d (R&O adopted Apr 30, 2026: NGSO-GSO performance-based framework replaces EPFD in 10.7-12.7, 17.3-18.6, 19.7-20.2 GHz; backstops: 3% time-weighted avg throughput degradation (ACM), 0.1% absolute increase in link unavailability, -10.5 dB I/N 80% for non-ACM; mandatory good-faith coordination; ITU EPFD obligations unchanged; no aggregate limits)

https://www.newamerica.org/insights/leo-satellites/chapter-i-fueling-connectivity-from-space-spectrum-sharing-and-coexistence/ (2023 R&O inter-round: earlier-round priority up to 10-year sunset; same-round default split)

# Note: FCC 5-year deorbit rule

URLs (index, read 2026-10-19):

https://docs.fcc.gov/public/attachments/DA-25-256A1.pdf (Second R&O in FCC 22-74, IB Docket Nos. 18-313 and 22-271: LEO region below 2,000 km, uncontrolled atmospheric reentry -> dispose ASAP but no later than 5 years after mission end; replaces 25-year guideline; 2-year transition)

https://advanced-television.com/2022/09/30/fcc-adopts-5-year-satellite-deorbiting-rule/ (adopted Sept 29/30, 2022; applies to US-licensed and foreign market-access satellites)

# Note: FCC SCS framework

URLs (index, read 2026-10-19):

https://www.tlp.law/2024/03/20/fcc-releases-adopted-scs-item/ (R&O+FNPRM, FCC 24-28, GN Docket 23-65, adopted Mar 14, 2024 (released Mar 15): secondary bi-directional MSS allocation; SCS only where terrestrial licensee(s) holding all licenses in a Geographically Independent Area lease spectrum to satellite operator; interim 911 routing)

https://www.satellitetoday.com/connectivity/2024/02/23/fcc-releases-the-draft-order-for-its-satellite-to-cell-regulatory-framework/

# Note: FCC bonds, fees, milestones

URLs (index, read 2026-10-19):

https://www.federalregister.gov/documents/full_text/html/2021/03/01/2020-28907.html (FCC 15-167, IB Docket 12-267: escalating surety bond $3M GSO / $5M NGSO, payable if milestones missed)

https://docs.fcc.gov/public/attachments/FCC-21-110A1.txt (fee schedule: NGSO space station $15,050/system/call sign; small-sat NGSO $2,175; market access NGSO $15,050; GSO $3,555)

https://www.insideglobaltech.com/2025/11/11/fcc-proposes-space-modernization-for-the-21st-century-key-highlights-for-industry-and-stakeholders/ (Space Modernization NPRM 2025 PROPOSED: eliminate GSO milestones; NGSO: 1 sat in 7 yrs, 10% in 9, 50% in 12, 100% in 14; de-escalating bond to $0 at 90% for >=200 sats; 20-yr license terms)